Characteristics of butterfly valves and service fluid

1. Title and characteristics of service fluid.

CPC Blend crude oil:

| Density, API | 57.2 -26.79 |
| --- | --- |
| Density at 20°С, kg/m3 | 730-890 |
| Density at 15°С, kg/m3 | 754.08 -893.44 |
| Viscosity at 20°C, сSt | 1.0 -6.0 |
| Sulfur, wt% | up to 1.8 |
| Chlorides, mg/dm3, not higher than | 100 |
| Water, wt%, not greater than | 0.5 |
| Mechanical impurities, wt%, not greater than | 0.05 |
| Paraffin, wt%, up to | 6.6 |
| Pour point, °С | -3 |
| Freezing point, °C | -18 |
| Closed-cup flash point, °С, max | 0 |
| Vapor pressure, kPa, not higher than | 66.7 |
| Hydrogen sulfide, 1/M (ppm), not higher than | 10 |
| Mercaptans, 1/M (ppm), not higher than | 30 |
| Total acid number (TAN) mg KOH/g | 0.2 |
| Product temperature, ºС | 5 to 60 |

2. Butterfly valve control method - bare stem butterfly valve without gearbox is required. Control with manual detachable lever. Bare stem must be sunk/flush with control head to protect control mechanism from impact on tanker hull. Valve control head must not have protruding elements. Head must have OPEN/CLOSED position

3. Butterfly valve external coating – epoxy paint, minimum thickness 175 micron.

4. Butterfly valve type – wafer-style butterfly valve

5. Butterfly valve seal – Viton (preferred)

6. Butterfly valve weight- not to exceed 115 kg.

7. Operating pressure 19 bar

Butterfly valve will be installed on marine hoses and serve in marine environment.

Package must include studs for fastening butterfly valve (PTFE coated, 1” x 247 mm), flange gasket (O-ring), socket wrench and control lever.