Task Order for

Purchase of offshore breakaway coupling

| Item # | Name of parameter | Value of parameter |
| --- | --- | --- |
| 1 | Operation/installation location | Black Sea, SPM of CPC-R Marine Terminal, external and internal sleeves of SPM floating hoses |
| 2 | Weather criteria of operation | Wind (1 min. at height of 10 m) - 36.9 m/s Wave height characteristic (over 100 years) - 6.95 m Wave period characteristic (over 100 years) -11.3 s Surface current (max.)* - 54 cm/s Half depth current (max.) 47см/ Bottom current (max.)* - 41 cm/s Baseline current - 1 m/s Water temperature (Centigrade) from -2° to + 26° Air temperature (Centigrade) from -24° to +39° Mooring operations, hosing and operation of auxiliary vessels of CPC-R Marine Terminal are determined by weather conditions: a) Mooring and hosing operations shall be performed in up to 2.5 m seas b) Auxiliary vessels of the Terminal can be operated and loading operations can be performed in up to 3.0 m seas. c) Tanker can be instructed to leave SPM in the event of the following conditions or forecast of such conditions at the instruction of CPC-R PSCI and/or CPC-R mooring master: 1. Wave height more then 4 meters; 2. Threat of floating hoses damage; 3. Wind velocity more then 23 m/sec. 4. Tension of the mooring hawser 100 tons and more. |
| 3 | Operating medium( oil) | Density , API at least 26.79 Density , at 20°С, kg/m3- not more than 890 Density , at 15°С, kg/m3- not more than 893.4 Sulfur total mass, % less than 1.8 Maximum chloride salts concentration, mg/dm3 - 100 Maximum water content, mass % 0.5 Maximum water content, mass % 6.6 Crude oil pour point, °С, less than -3.0 Initial boiling point, °С, more than 35 Ignition point, °С, less than 21 Saturated vapor pressure, kPa (mm Hg), not more than 66.7 (500) Hydrogen sulfide, 1/M .(ppm) below 10.0 Mercaptan, 1/M .(ppm,) below 30.0 Cut yield - at up to 200°С at least 21 - at up to 300°С at least 42 |
| 4 | Breakaway coupling type | Double closure, reed valve |
| 5 | Nominal Bore | 16 Inch |
| 6 | Pressure (bar): Operating Testing during assembly | 19.0 28.5 |
| 7 | Tripping threshold | at a tension of 35 tons at an internal pressure of 27.3 bars |
| 8 | Product operating temperature, °С | -20°С to +82°C |
| 9 | Time to close | Not more than 32 seconds |
| 10 | Body material | ASTM A105N steel or equivalent |
| 11 | Coupling material | AISI 304L steel or equivalent |
| 12 | Reeds material | ASTM A747 CB7CU-1 (H1075 GRADE) or equivalent |
| 13 | Shear pin material | TITANIUM Ti-6Al-4V or equivalent |
| 14 | Flanges | ANSI В 16,5 class 150 |
| 15 | Flange fasteners | Corrosion resistant PTFE coated studbolts |
| 16 | Breakaway coupling floats | Floatation over 550 kg |
| 17 | Expected service life, years | At least 20 years with regular maintenance after 48 months of service |
| 18 | Supporting documents | Passport, manual for operation, installation, dismantling and storing. Breakaway couplings shall be manufactured and tested under supervision of ABS or Russian Register of Shipping with issue of a relevant certificate. |